Darlington County Codes Enforcement

2009 International Energy Conservation Code

Air Barrier & Thermal Barrier Inspection Checklist

Air barrier & thermal barrier

Exterior thermal envelope insulation for framed walls is installed in substantial contact and continuous alignment with building envelope air barrier.  										Date: ________________

Breaks or joints in the air barrier are filled or repaired. 						Date: ________________

Air-permeable insulation is not used as a sealing material.  						Date: ________________

Ceiling /attic

Air barrier in any dropped ceiling/soffit is substantially aligned with insulation and any gaps are sealed.  	Date: ________________

Attic access (except unvented attic), knee wall door, or drop down stair is sealed.  			Date: ________________

Walls:  Corners and headers are insulated.  Junction of foundation and sill plate is sealed.  		Date: ________________

Windows and doors:  Space between window/door jambs and framing is sealed. 			Date: ________________

Rim joists:  Rim joist are insulated and include an air barrier.  						Date: ________________

Floors: (Including above garage and cantilevered floors).  Insulation is installed to maintain permanent contact with underside of subfloor decking. 										Date: ________________

Crawl space walls: (Encapsulated crawl spaces only)

Insulation is permanently attached to walls								Date:  ________________

Exposed earth in unvented crawl space is covered with a Class I vapor retarder with overlapping joints.	Date:  ________________

Shafts, penetrations: Duct shafts, utility penetrations, knee walls and flue shafts opening to exterior or unconditioned space are sealed.												Date:  ________________

Narrow cavities:  Batts in narrow cavities are cut to fit, or narrow cavities are filled by sprayed/blown insulation.  Date: ___________

Garage separation:  Air sealing is provided between the garage and the conditioned space.		Date:  ________________

Recessed lighting:  Recessed light fixtures are air tight, IC rated and sealed to drywall.  			Date: ________________

Plumbing and wiring

Insulation is placed between outside and pipes.							Date: ________________

Batt Insulation is cut to fit around wiring and plumbing, or sprayed/blown insulation extends behind piping wiring.  Date: _________

Shower/tub on exterior wall:  Showers and tubs on exterior walls have insulation and air barrier separating them from exterior walls.												Date: ________________

Electrical/phones boxes on exterior wall:  Air barrier extends behind boxes or air sealed-type boxes are installed.  Date: _________

Common wall:  Air barrier in installed in common wall between dwellings.				Date: ________________

HVAC register boots:  HVAC register boots that penetrate building envelope are sealed to subfloor or drywall.  Date: ____________

Fireplace:  Fireplace walls include air barrier.  This Includes but not limited to false fireplaces for vent-less gas logs. Date: _________

Visual Inspection Conducted by: _________________________________        	Today’s Date: _______________________________