Development Services – Engineering Land Development

311 Vernon Street

Roseville, CA  95678 (916) 774-5339

# PUBLIC IMPROVEMENTS REQUIREMENTS

TREE PRESERVATION:

(Notes required if grading is part of the tentative approved project and trees are conditioned to

be saved)

- GRADING AROUND TREES TO BE LIMITED TO OUTSIDE THE DRIPLINE.  ANY TREE WELL GRADED SHOULD BE DRAINED TO SOME OUTFALL OR SWALE.

- THE FOLLOWING CONDITIONS SHALL APPLY TO ENSURE THE PRESERVATION OF TREES:

  - EACH TREE OR GROUP OF TREES TO BE SAVED SHALL BE FENCED BY A CHAIN-LINK FENCE PRIOR TO ANY GRADING OR MOVEMENT OF HEAVY EQUIPMENT OR ISSUANCE OF ANY PERMITS.

  - FENCING SHALL BE LOCATED ONE FOOT (1') OUTSIDE THE DRIPLINE OF THE TREE OR TREES.

  - SIGNS SHALL BE POSTED ON ALL SIDES OF SAID FENCES STATING:

WARNING:

THIS FENCE SHALL NOT BE REMOVED

OR RELOCATED WITHOUT WRITTEN

AUTHORIZATION FROM THE

PLANING DEPARTMENT

- THE GRADING PLAN FOR THE PROJECT HAS BEEN DESIGNED FOR NO GRADING TO OCCUR WITHIN THE DRIPLINE OF ANY OAK TREE UNLESS SPECIFICALLY APPROVED BY PLANNING DEPARTMENT AND SHOWN ON THESE PLANS. NO GRADES SHALL BE MODIFIED WITHOUT THE APPROVAL OF THE CIVIL ENGINEER AND THE CITY OF ROSEVILLE.

- NO TRENCHING SHALL OCCUR BENEATH THE DRIPLINE OF ANY OAK TREE TO BE SAVED UNLESS STATED ON THESE PLANS "TRENCHING UNDER THIS TREE IS APPROVED".

- PARKING OF VEHICLES AND EQUIPMENT OR STORAGE OF MATERIALS UNDER THE DRIPLINE OF TREES SHALL NOT OCCUR AT ANY TIME.

E:\Develop\Standard Notes\Tree Preservation	REV. 2/23/05